| Title |  |  |  |  |
| --- | --- | --- | --- | --- |
| Comprehensive study on the sand spit elongation at tidal inlets in Vietnam Nghiên cứu toàn diện về cơ chế phát triển kéo dài của doi cát ở cửa sông Việt Nam |  |  |  |  |
| RONPAKU Fellow |  |  |  |  |
| Name | Duc Anh Quang Nguyen |  |  |  |
| Position | Doctor | Institution |  | Institute of Civil Engineering |
| Japanese Advisor |  |  |  |  |
| Name | Hitoshi Tanaka |  |  |  |
| Position | Professor | Institution | Tohoku University |  |
| Vietnamese Advisor |  |  |  |  |
| Name | Nguyen Trung Viet |  |  |  |
| Position | Professor | Institution | Thuyloi University |  |

Tidal inlets along the central coast of Vietnam are located in a micro-tidal, wave-dominated coastal environment with wide seasonal variations in river flow and wave climate. They often suffer from periodic entrance shoaling and/or mouth closure – barrier breaching. As the main reasons for the increase in the risk of coastal flooding and the obstruction of navigation, the migration or closure of tidal inlets has caused a lot of problems for the socio-economic development in the region. Thus, it is essential to study and analyze the morphological processes of the inlets and build up scientific foundations for future development plans. This study investigates on the sand spit elongation phenomenon at 6 tidal inlets in Central Vietnam by employing both satellite image analysis and numerical methods. From the results of this study, it is found that among all the factors affecting the development of the sand spits, the elongation mechanism qualitatively and quantitatively determines the morphological change of sand spits. Based on the conservation equation for sand and the data analysis from 6 case studies, a new simple empirical formula for the prediction of the sand spit elongation rate is developed. The equation was calibrated using the comprehensive data for the central coast of Vietnam, Japanese coastal areas, and 3 sand spits in Senegal, U.S, and Sweden. In all case studies, the sand spit growth rate Rs is inversely proportional to the sand spit width Bs.

By assuming that the magnitude of LST Q along the central coast of Vietnam receives a value in the above-limited range and the sum of (DB + DC) equals 10 m, the sand spit growth rate is obtained as . This value can be applied to predict the sand spit growth rate using a single unknown parameter for the inlets in the center of Vietnam.

| Photos |  |
| --- | --- |
| Field visit to Halong Bay, Vietnam, with Prof. Hitoshi Tanaka and Prof. Nguyen Trung Viet | Online examination (1st examination) |

NOTE: Please write the abstract in English at the maximum 2 pages, and then send the abstract together with the photos to which you upload this file as mail attachments. The abstract and your photos will be posted on JSPS’s website.